Sukkur IBA University

Department of Computer Science

Applied Physics, Spring 2024

Lab-08: Verification of Kirchhoff’s Current Law

Instructor: Engr. Asif Ali

Note: Submit this lab hand-out in the next lab with attached solved activities and exercises

Submission Profile

Name:

Submission date:

Marks obtained:

Receiving authority name and signature:

Comments:

________________________________________________________________________________

Instructor Signature

- Lab Learning Objectives:

- To get hands on practice to design & analyze the linear circuits on NI Elvis-II and Multisim.

- To understand & verify the KCL.

- Gain proficiency in using Lab equipment & software for simulation.

- Lab Hardware and Software Required:

- NI Elvis-II Kit

- Resistors

- Connecting wires

- PC

- Multisim

Background Theory:

In 1847, Gustav R. Kirchhoff gave two fundamental laws which are very useful in electric circuits. These laws are more comprehensive than Ohm’s Law and used for solving electrical network which may not be easily solved by the Ohm’s Law.

Kirchhoff’s Current Law:

Kirchhoffs Current Law or KCL, states that the “total current or charge entering a junction or node is exactly equal to the charge leaving the node as it has no other place to go except to leave, as no charge is lost within the node“.

Sign Convention: The current entering in a junction is assigned positive sign and the current leaving the junction is assigned negative sign.

Here, the 3 currents entering the node, I1, I2, I3 are all positive in value and the 2 currents leaving the node, I4 and I5 are negative in value. Then this means that we can also rewrite the equation as; I1 + I2 + I3 – I4 – I5 = 0

- Lab Activities:

- KCL VERFICATION:

- Design the circuit as shown in Fig. 1. Take the resistors that are available in the laboratory as per requirements & measure their ohmic value and put the readings in Table. 1. Also, find their nominal value using color coding.

- Turn on the 5V power supply from NI-ELVIS-II.  Now measure the required values through ammeter and fill the corresponding Table. 2.

- Put the readings of measured currents i.e. I1, I2 & I3 in equation.1 to verify the equation of KCL.

 Perform the same activity for the circuit shown in Fig. 2. Put the corresponding values in Table

to verify KCL in the given circuit.

I

1

I

2

I

3

FIGURE. 1

WORKSHEET

- KCL VERIFICATION:

- TABLE 1:

| RESISTANCE | COLOUR CODED VALUE in ohm | MEASURED ohm |
| --- | --- | --- |
| R1 | 220 | 230 |
| R2 | 470 | 537 |
| R3 | 3300 | 3244 |
| R4 | 150 | 178 |

- TABLE 2:

| CURRENT (mA) | CALCULATED (mA) | MEASURED (mA) |
| --- | --- | --- |
| I1 | 0.00624 | 0.006398 |
| I2 | 0.00545 | 0.005601 |
| I3 | 0.00080 | 0.0007975 |

I1					I2						I3

Lab Exercise

Task-01: Design the following circuit on Multisim and fill the Table-03

FIGURE. 2

I

1

I

2

I

3

- TABLE 3:

| CURRENT (mA) | CALCULATED | MEASURED |
| --- | --- | --- |
| I1 | 0.007891 | 0.00751 |
| I2 | 0.000945 | 0.000929 |
| I3 | 0.00694 | 0.00700 |