Sukkur IBA University

Department of Computer Science

Applied Physics

BS (CS)-II

Spring 2024

Lab # 03: DC Series circuit

Instructor: Engr: Asif Ali

Note: Submit this lab hand-out in the next lab with attached solved activities and exercises

Objectives:

- To get hands on practice to design & analyse the linear circuits on NI Elvis-II.

- The objective of this experiment is to determine the equivalent resistance of series circuit. You will apply various voltages across the resistor (or circuit) using a power supply and measure the current I through the resistor and the voltage V across the resistor for each setting of the power supply. From a plot of V vs. I, you will determine the resistance. You will compare the measured value of the resistance with the manufacturer's value.

Lab Hardware and Software Required:

- NI Elvis-II Kit

- Resistors

- Connecting wires

- PC

- Multi-sim

Background Theory:

Combinations of Resistors

Resistors can be combined in simple circuit arrangement that can increase the resistance in the circuit. This arrangement is called series circuit. Figure 1 illustrates resistors are connected in series.

When charges move in a conductor, there must be a potential difference across the conductor, and there must be a complete path leading away from and back to the source of emf. In the series arrangement shown in Figure 1, the current I in the circuit goes through each resistor. If we compute the potential drop ΔV1 across R1 using Ohm's Law, it is ΔV1 = IR1: Likewise, the drop across R2 is ΔV2 = IR2: The potential drop across both resistors is ΔV =ΔV1 +ΔV2 which is equal to ". One can think of the applied voltage "being divided between the two series resistors R1 and R2. Using algebra, the relationships for determining equivalent resistance Req for resistors in series and/or parallel can be derived. Series Req = R1 + R2 + ……… (1) The equivalent resistance is the sum of the individual resistances.

Lab Activities:

Solve the below series circuit. To find unknown quantities as mentioned in worksheet.  Choose the values of resistors of your own choice.

Figure 2

Procedure: Determining Equivalent Resistance-Series Arrangement

- Connect the two resistors in a series arrangement.

- Supply 5V to the circuit

- Find the values of four resistors

- Find the voltage and current across R1, R2, R3, and R4

Worksheet

Table 1

| Resistors | R1 | R2 | R3 | R4 |
| --- | --- | --- | --- | --- |
| R (Nominal) | 100 ohm | 50 ohm | 100 ohm | 40 ohm |
| R (Actual) | 92 ohm | 59 ohm | 102 ohm | 38 ohm |

Table 2

| Values | I | V1 | V2 | V3 | V4 |
| --- | --- | --- | --- | --- | --- |
| Calculated |  |  |  |  |  |
| Measured |  |  |  |  |  |

Lab Exercise:

Solve any 2 examples from textbook on A4 size blank papers, to be attached with the lab handout and simulate using Multi-sim. The results and circuit snapshots should be attached with the lab handout.