Sukkur IBA University

Department of Computer Science

Applied Physics

Lab # 04: Parallel circuit

Instructor: Engr: Asif Ali

Note: Submit this lab hand-out in the next lab with attached solved activities and exercises

Submission Profile

Name: 							Submission date:

Enrolment ID:					Receiving authority name and signature:

Comments:

________________________________________________________________________________

Instructor Signature

Submission Profile

Name: 							Submission date:

Enrolment ID:					Receiving authority name and signature:

Comments:

________________________________________________________________________________

Instructor Signature

Objectives:

- To get hands on practice to design & analyse the linear circuits on NI Elvis-II.

- The objective of this experiment is to determine the equivalent resistance parallel circuit and also find the current across different resistors in the parallel circuit.

- You will apply various voltages across the resistor (or circuit) using a power supply and measure the current I through the resistor and the voltage V across the resistor for each setting of the power supply. From a plot of V vs. I, you will determine the resistance.

- You will compare the measured value of the resistance with the manufacturer's value.

Lab Hardware and Software Required:

- NI Elvis-II Kit

- Resistors

- Connecting wires

- PC

- Multisim

Background Theory:

Combinations of Resistors

Resistors can be combined in simple circuit arrangements that increase or decrease the over- all resistance in the circuit. These arrangements are called series and parallel circuits. Following Fig. shows the resistors in a parallel arrangement.

Figure 1

In order to charges move in a conductor, there must be a potential difference across the conductor, and there must be a complete path leading away from and back to the source of emf. In the parallel arrangement shown in Figure 1. the current divides at the junction A and recombines at junction B. Therefore, the current through R1 and R2 will be different. Notice that in this case ΔV = ΔV1= ΔV2: That is, the potential drop across each resistor is the same. Using algebra, the relationships for determining equivalent resistance Req for parallel can be derived. Parallel

1/Req = 1/R1 + 1/R2 + ……. 1/Rn

The reciprocal of the equivalent resistance is the sum of the reciprocals of the individual resistances.

Lab Activities:

Solve the below series circuit. To find unknown quantities as mentioned in worksheet.  Choose the values of resistors of your own choice

Figure 2

Table 1

| Resistor | Band 1 |  | Band 2 |  | Band 3 |  | Resistance | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Color | Value | Color | Value | Color | Value | Ω | Color | Tolerance % |
| 1 |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |

Procedure: Determining Equivalent Resistance-Series Arrangement

- Connect the two resistors in a parallel arrangement.

- Supply 5V to the circuit

- Find the values of four resistors

- Find the voltage and current across R1 and R2

Worksheet

Table-2

| Resistors | R1 | R2 |
| --- | --- | --- |
| R (Nominal) |  |  |
| R (Actual) |  |  |

Table-3

| Values | V | I1 | I2 |
| --- | --- | --- | --- |
| Measured |  |  |  |
| Calculated |  |  |  |

Lab Exercise

1. Solve the following circuits and attach solve results on A4 page, MultiSim result and also Elvis Kit result.

Task-01

| Resistor | Band 1 |  | Band 2 |  | Band 3 |  | Resistance | Band 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Color | Value | Color | Value | Color | Value | Ω | Color | Tolerance % |
| 1 |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |

Procedure: Determining Equivalent Resistance-Series Arrangement

- Connect the two resistors in a parallel arrangement.

- Supply 5V to the circuit

- Find the values of four resistors

- Find the voltage and current across R1, R2, and R3

Worksheet

Table-2

| Resistors | R1 | R2 | R3 |
| --- | --- | --- | --- |
| R (Nominal) |  |  |  |
| R (Actual) |  |  |  |

Table-3

| Values | V | I1 | I2 | I3 |
| --- | --- | --- | --- | --- |
| Measured |  |  |  |  |
| Calculated |  |  |  |  |

2. Solve the following circuits and attach solve results on A4 page, MultiSim result and also Elvis Kit result.

Task-02