# Clipper Circuits

A clipper circuit is consisted of a diode and a resistor. It removes part of input signal based on requirement without distorting the remaining part of input signal. It can:

- Clip portion or entire positive half of input signal

- Clip portion or entire negative half of input signal

- Clip portions of both positive and negative half of input signal.

- The protection of the electronic devices can be achieved by using the clipper circuits.

- It is an application of diode.

- One of the basic clipping device is the half wave rectifier

## Types of clippers

The various types of clippers are as follows:

- Series Clippers: In series clippers, the diode is connected in series with the output load resistance.

- Series positive clipper

- Series positive clipper with bias

- Series negative clipper

- Series negative clipper with bias

- Shunt Clippers: In shunt clippers, the diode is connected in parallel with the output load resistance.

    - Shunt positive clipper

    - Shunt positive clipper with bias

    - Shunt negative clipper

    - Shunt negative clipper with bias

Clipper with bias

Bias is constant voltage or current added to an input signal or circuit. Sometimes it is desired to remove a small portion of positive or negative half cycles. In such cases, the biased clippers are used. An extra element called battery is used in clippers.

# Clamper Circuits

A clamper circuit is constructed of a diode, resister, and a capacitor. It shifts the waveform to a different DC level without changing the appearance of the applied signal.

- Negative Clamper

- Positive Clamper